# DLC DMA System
Specification

Document Author: Jinniu

Document Owner: Jinniu

Document Revision History:

| Revision | Date | Author | Comments |
| --- | --- | --- | --- |
| 0.1 | 02/23/2019 | Jinniu | Initial Version |

Chapter 1: Introduction	5

Chapter 2: Concepts	5

2.1 Transaction	5

2.2 Descriptor	5

2.3 Actor	5

List of Actors	5

2.4 Types of DMA	6

Local	6

Remote	7

Host	8

Chapter 3: Synchronization	13

3.1 Sync Flag	14

3.2 Memory Model	14

Chapter 4: Completion	15

4.1 Globally Complete	15

4.2 Observably Complete	15

4.3 Partially Complete	15

4.4 Engine Complete	15

Local DMAs	15

Remote DMAs	16

Host2chip DMAs	16

Chip2host DMAs	16

Chapter 5: Descriptor	16

Chapter 6: DMA Transfer Properties	18

6.1 DMA Length	18

6.2 0-Length DMAs	18

6.3 Strided DMAs	19

Stride	19

Steps per stride	19

6.4 Supported DMA Sources and Destinations	19

Local and Remote DMAs	19

Host2Chip DMAs	19

Chip2Host DMAs	20

Magic DMAs	20

Chapter 7: Chip DMA Subsystem	20

7.1 Overview	20

Contention Points	21

7.2 Functional Descriptions	22

Supported Transfer Types	22

Modeling Pull Transfer Model Using Push-Push	22

Software Synchronization Primitives	22

Modeling Request/Response Using Acknowledgement Messages	23

Chapter 8: OCN Protocol	23

8.1 Concepts	23

Completion	23

8.2 Components	23

Nodes	23

Channels	24

8.3 Transactions	25

8.4 Address Space	27

8.5 Ordering	27

8.6 Channel Dependencies	28

Descriptor Input Channel	28

Command Input Channel	28

Data Input Channel	28

Response Input Channel	28

Message Input Channel	29

8.7 Other Dependency Requirements	29

# Introduction

# Concepts

## Transaction

A DMA Transaction is an asynchronous transfer of data between two memories. One of the memories must be on-chip.

## Descriptor

A DMA Descriptor is a data structure that encodes information to perform a single DMA Transaction. A general form of DMA Descriptor is used for Local, Remote and some Host DMAs, while an additional Host DMA Descriptor is used for Host DMAs.

## Actor

A DMA Actor is a hardware resource that participates in a DMA Transaction.

- Requestor: A hardware resource that emits a given DMA Descriptor, usually a core.

- Source Memory: A memory resource where data is read.

- Destination Memory: A memory resource where data is written.

- Engine: A hardware resource that receives a DMA Descriptor and execute the DMA Transaction.

- Sync Flag: A special-purpose memory for synchronization, sitting in a core.

- Bridge: A hardware resource where data flows from source memory to destination memory. One of the memories must be off-chip. Descriptors do not pass through bridges. There can be multiple bridges on the path, and one must be an engine.

All DMA Transactions must always have a single Requester, a single Source Memory, at least one Destination Memory, a single Engine, an optional Source Sync Flag, and optionally up to two Destination Sync Flags. If either Source or Destination Memory is off-chip, one or more Bridges are involved.

### List of Actors

#### Requesters

Cores (XYS, DHB), host software (via HTT)

#### Source / Destination Memories

Shared memories

- FXC (Cmem)

- HBM

XYS memories

- Imem (destination only, will raise a fatal error if read)

- Smem

- Vmem

DHB memories

- Smem

- Dmem

- DImem

- DVmem

- HMF (destination of Infeed Host DMA only, not addressable by DMA Descriptors)

Logical memories within chip shared memories (data structures for consuming / producing host infeed / outfeed data)

- Chip Infeed Buffers (arbitrarily and statically located in a memory that works as both a source and destination; feeds cores)

  - For Local DMAs, they are source only

  - For Hardware Infeed DMAs, they are destination only

  - Circular buffer with head and tail pointers

  - One per Hardware Infeed DMA Descriptor Queue

Logical memories within host main memory (data structures for producing / consuming chip infeed / outfeed data)

- Host Infeed Buffers (arbitrarily and dynamically located in host memory, fed by host user software)

  - For Hardware Infeed DMAs, they are source only

- Host Outfeed Buffers (arbitrarily and dynamically located in host memory, feeds host user software)

  - For Hardware Outfeed DMAs, they are destination only

- Host Direct-Write Buffers (arbitrarily and dynamically located in host memory, fed by host user software)

  - For Direct-Write DMAs, they are source only

- Host Debug Buffers (arbitrarily and dynamically located in host memory, feeds host user software)

  - For Debug DMAs, they are destination only

#### Engines

Local DMAs

- HBM, where HBM is the destination

- FXC, where Cmem or Vmem is the destination

- XYS, where Imem or Smem is the destination

- DHB, where Dmem, Smem, DImem, DVmem or HMF is the destination

Remote DMAs

- Local LYP Router

Host DMAs

- HTT

#### Bridges

LYP Routers (Remote DMAs)

HTT (Host DMAs)

#### Sync Flag Memories

XYS Sync Flag Memories

DHB Smem

HTT Sync Flag Memories (for certain Host DMAs only)

## Types of DMA

### Local

A Local DMA Transaction is for data transfers within a single chip, implemented using KLM protocol, always pull-based from a DMA engine.

- Requester: local

- Source Memory: local

- Destination Memory: local

- Engine: Destination Memory

- Sync Flag (optional): local

- Bridge: N/A

Fig. logic and timing diagram of Local DMA

### Remote

A Remote DMA Transaction is for data transfers between a local chip and a remote chip, implemented using KLM and LYP protocol combined, always push based from a local chip

- Requester: local

- Source Memory: local

- Destination Memory: remote

- Engine: local LYP router

- Sync Flag (optional): local and/or remote

- Bridge: local and remote LYP routers

Fig. logic and timing diagram of Remote DMA

### Host

A Host DMA Transaction is for data transfers between a single chip and the host. Each Host DMA Transaction is associated with a Host DMA Descriptor Queue. There are two primary sub-types of Host DMA, and two additional special sub-types that are not for mission mode.

#### Host2chip

A Host2chip DMA Transaction transfers data from the host to a chip, which is further categorized into the below two sub-types.

- Hardware Infeed

A Hardware Infeed DMA Transaction allows the host to copy data from an arbitrary and dynamically allocated Host Infeed Buffer into one of several fixed and statically-allocated buffer locations on-chip (Chip Infeed Buffer).

Protocol: OCN and PCIe protocol combined

Push-pull Semantic: always push-based from the host

- Requester: host (software)

- Source Memory: host (arbitrary Host Infeed Buffer)

- Destination Memory: local (fixed Chip Infeed Buffer)

- Engine: HTT

- Sync Flag (optional): local

- Bridge: HTT

- Direct-Write DMA

A Direct-Write DMA Transaction allows the host to copy data to any one or more locations in local chip shared memories (FXC or HBMs). This can be used to implement Software Infeed, chip memory initialization, load chip program, etc.

Protocol: OCN and PCIe protocol combined

Push-pull Semantic: always push-based from the host

- Requester: host (software)

- Source Memory: host (arbitrary Host Direct-Write Buffer)

- Destination Memory: local (arbitrary locations in chip shared memories)

- Engine: HTT

- Sync Flag (optional): local

- Bridge: HTT

Fig. logic and timing diagram of Host2chip DMA

#### Chip2host (Hardware Outfeed)

A Chip2host DMA Transaction (or Hardware Outfeed DMA Transaction) allows the host to copy data from arbitrary on-chip memory locations into an arbitrary and dynamically allocated buffer location in the host memory (Host Outfeed Buffer). This is the only DMA Transaction type that requires two Requesters, the host and a core on the chip.

Protocol: OCN and PCIe protocol combined

Push-pull Semantic: always pull-based from the host and always push-based from the chip, which is essentially a handshake mechanism.

- Requester: host (software) and local (on-chip core)

- Source Memory: local (arbitrary locations in chip shared memories)

- Destination Memory: host (arbitrary Hardware Outfeed Buffer)

- Engine: HTT

- Sync Flag (optional): local

- Bridge: HTT

Fig. logic and timing diagram of Chip2host DMA

#### (Special Purpose) Magic DMA

A Magic DMA Transaction is a special sub-type of Host DMA that allows the host to “magically” inject an arbitrary Descriptor into the chip. A Magic DMA causes a Magic Child DMA Transaction to occur as if a core had requested it, i.e. it allows the host to emulate the behavior of a core. Magic Child DMAs can take the form of any Local or Remote DMA and is also possible to be a part of a Hardware Outfeed transaction with the host participating as well. Magic DMAs might be used for hardware or software debug purposes. Although Magic DMA itself is not a true DMA since there is no explicit Source / Destination Memory, the resulting Magic Child DMA would have them.

Protocol: PCIe for Magic DMA, and based on injected Descriptor for Magic Child DMA

Push/Pull Semantic: N/A for Magic DMA, and based on injected Descriptor for Magic Child DMA

- Requester: host (software)

- Source Memory: N/A

- Destination Memory: N/A

- Engine: HTT

- Sync Flag (optional): N/A

- Bridge: HTT

Fig. logic and timing diagram of Magic DMA

#### (Special Purpose) Debug DMA

A Debug DMA Transaction is a special type of Host DMA that transfers hardware debug information from various units in the chip to the host. Essentially, it resembles a special kind of Chip2host (Hardware Outfeed) where the data does not come from architecturally visible memory state.

Protocol: a combination of Debug Bus, CSR Ring and PCIe

Push/Pull Semantic: always pull-based from the host

- Requester: host (software)

- Source Memory: local (on-chip Debug Bus that is not architecturally visible to core software)

- Destination Memory: host (arbitrary Host Debug Buffer located in host main memory)

- Engine: HTT

- Sync Flag (optional): N/A

- Bridge: HTT

Fig. logic and timing diagram of Debug DMA

# Synchronization

Each DMA Transaction is executed asynchronously with respect to any software/control thread and to each other. Distinct DMA Transactions are completely independent of each other and there is no specific ordering between their progress and/or completions. The constituent memory reads and writes in one transaction may also interleave in an arbitrary order with accesses of other transactions. Last, read and write within a single transaction may occur out of order. Following facts may contribute to this behavior:

- Variable delay from the time a DMA Descriptor is emitted to the time it is received at the appropriate Engine

- Out-of-order memory read and write

- Arbitration of shared interconnect resources

- The possibility of multi-path routing

If the software requires a guaranteed DMA ordering of data movement, it must rely on an explicit synchronization through Sync Flags. Any access to the affected memory that is beyond Partial Completion progress indicated by the associated Sync Flags might observe the memory in an inconsistent or undefined state due to RAW, WAR or WAW hazards.

## Sync Flag

Sync Flag is an atomically accessed memory to synchronize control threads between cores to each other and otherwise asynchronized DMA transactions. A Sync Flag contains two fields:

- Count (Bit [30:0]): a 31-bit signed integer to reflect numbers of DMA transactions executed by a DMA engine in a unit of 512B, updated on a DMA completion.

  - A DMA engine can increment or set, but not decrement the count due to the fact that there is no negative-length DMA.

  - A core can increment, decrement or set the count. Besides, a parent core can read or do signed comparison.

- Done (Bit [31]): a flag that indicates a completion of a DMA transaction.

  - A DMA engine always sets.

  - A core atomically read, write or wait for the done bit.

  - Architecturally visible only through ISA or to a DMA engine.

  - Does not transfer through DMA transactions.

A Sync Flag update is an atomic write operation by a DMA engine or a core to a single Sync Flag in any Sync Flag memory. If a DMA engine’s Sync Flag update increments a Sync Flag and causes the count to overflow to a negative value, a program error is raised. Note that a 0-length DMA is supported. In this case, the count field is not updated but the done bit can be optionally set.

A Sync Flag check is an atomic read operation of a single Sync Flag that may only be performed by its parent core that contains the Sync Flag memory. The parent core can judge the completion with respect to Source or Destination Memory. A core cannot perform such a check for another core, and an engine cannot perform such a check at all.

## Memory Model

The following ordering guarantees (memory model) are given between a specific control thread on a core and a specific DMA Transaction.

Emitting a DMA Descriptor from a Requestor (control thread), e.g. a Sync Flag update, is a write-release operation. Receiving and beginning execution of a DMA Descriptor in the engine, e.g. a Sync Flag check, is a read-acquire. The Requestor control thread thus synchronizes with the start of the DMA Transaction (a DMA thread that is executed by the Engine). This implies that the DMA Transaction observes the effect of any non-DMA memory load/store instructions that were executed by the control thread prior to emitting the DMA Descriptor.

Sync Flag check on the source Sync Flag avoids WAR hazard at the source memory; Sync Flag check on the destination Sync Flag avoids RAW/WAW hazards at the destination memory. Two control threads can also synchronize with each other using Sync Flags in this way without using a DMA Transaction.

# Completion

## Globally Complete

A DMA transaction is Globally Complete if and only if Sync Flag checks on the source Sync Flag and the destination Sync Flag both indicate “done”. Typically, it is not possible for any single core to do Sync Flag Checks on both the Source Sync Flag and the Destination Flag, so this concept is mostly a mental model.

## Observably Complete

It is possible for the Transaction to become Observably Complete before it is Globally Complete.

- Source Complete: if and only if Sync Flag checks on the source Sync Flag indicates “done”. The data may be safely overwritten in the Source Memory, even if the data is still in flight and has not yet become visible at the Destination Memory.

- Destination Complete: if and only if Sync Flag checks on the destination Sync Flag indicates “done”. The data may be safely read in the Destination Memory, and it always implies a Source Complete due to time causality, although Source/Destination Complete may happen out of order due to delays.

## Partially Complete

The DMA Engine monotonically increases the count value of the Source and Destination Sync Flags as incremental progress on the Transaction is made, through which the software can effectively observe a Transaction’s constituent reads and writes to occur in order. Software can safely and correctly access some DMA data that is part of a Partially Complete DMA Transaction that is still executing asynchronously in the background.

## Engine Complete

For Local DMAs, the DMA Engine can flush the Descriptor after its final source Sync Flag update and destination Sync Flag update are sent. It is defined differently for Remote / Host DMAs due to the absence of Source / Destination Sync Flag on the same chip.

### Local DMAs

For Local DMAs, the DMA Engine that is executing a Transaction may flush the DMA Descriptor from its internal state if and only if it has sent its final Source Sync Flag Update and final Destination Sync Flag Update. This does not require the Engine to wait for the Sync Flag Updates to actually become visible since Sync Flag Updates are guaranteed to eventually become visible. Therefore, a Transaction is Engine-Complete when the Transaction’s completeness is visible to the Engine but not necessarily other hardware units that are waiting on the Sync Flags.

### Remote DMAs

For Remote DMAs, the DMA Engine / Local Bridge (local LYP Router) that is executing a Transaction may flush the DMA Descriptor from its internal state if and only if it has sent its final Source Sync Flag Update and sent its final data packet to the Remote Bridge (remote LYP Router).

### Host2chip DMAs

For Host2chip DMAs, the DMA Engine (HTT) that is executing a Transaction may flush the DMA Descriptor from its internal state if and only if it has sent its final Destination Sync Flag Update and sent an interrupt to the host.

### Chip2host DMAs

For Chip2host DMAs, the DMA Engine (HTT) that is executing a Transaction may flush the DMA Descriptor from its internal state if and only if it has sent its final Source Sync Flag Update. Similarly, HTT may flush the Host DMA Descriptor from its internal state if and only if it has sent its final Queue Status Block update and then sent an interrupt to the host.

# Descriptor

A DMA Descriptor provides the control information needed to execute a single DMA that moves data from a source memory to a destination memory. All fields are aligned to the native 32-bit word. Software should never set any of these fields to a reserved value.

| header | [31:31] | trace_en | Generate a debug trace if trace is enabled. |
| --- | --- | --- | --- |
|  | [30:29] | dst_opcode | 0: write, 1: Reserved, 2: Accumulate for Cmem, compare and swap for Smem (must be 4B), 3: Read and Add if destination is Smem (must be 4B) |
|  | [28:26] | dst_core_id | dma_type == local/remote: 0: Reserved (self), 1: FXC/HBM for mem, HTT for Sync Flag, 2-3: XYS0-1, 4-7: DHB0-3, dma_type == Chip2Host: 0-7: Outfeed DMA Descriptor Queue 0-7 |
|  | [25:24] | dst_mem_id | core_id 1 (FXC/HBM): 0: HBM, 1: Reserved, 2: Cmem, 3: Reserved core_id 2-3 (XYS): 0: Vmem, 1: Smem, 2: Imem, 3: Reserved core_id 4-7 (DHB): 0: Dmem, 1: Smem, 2: Imem, 3: VImem |
|  | [23:23] | (reserved) | Should be 0 |
|  | [22:21] | src_opcode | 0: read, 1: Reserved, 2: Instruction memset, 3: Data memset Memset does not actually read the memory and directly generate source data from the Descriptor. |
|  | [20:18] | src_core_id |  |
|  | [17:16] | src_mem_id |  |
|  | [15:14] | dma_type | 0: local, 1: Chip2Host, 2: remote unicast, 3: remote multicast |
|  | [13:10] | (reserved) | Should be 0 |
|  | [9:0] | dst_id | dma_type == local/Chip2Host: ignored dma_type == remote unicast: destination chip_id dma_type == remote multicast: chip_id used by LYP |
| src_sync_flag | [31:16] | (reserved) | Should be 0 |
|  | [15:13] | src_sync_core_id |  |
|  | [12:0] | src_sync_flag_id | 0: null (no Sync Flag update), 1-0x1fff: Sync Flag offset into the associated Sync Flag memory |
| dst_sync_flag | [31:29] | dst1_sync_core_id |  |
|  | [28:16] | dst1_sync_flag_id |  |
|  | [15:13] | dst0_sync_core_id |  |
|  | [12:0] | dst0_sync_flag_id |  |
| length | [31:31] | granule | 0: dma_length = length * 512B, 1: dma_length = length * 4B |
|  | [30:0] | length | Unsigned, can be 0. When granule is 4B, dma_length must be less than or equal to 64B. Out-bound access causes a fatal error. |
| src_mem_addr | [31:31] | granule | Granule: 0: mem_offset indicates a 512B word, 1: mem_offset indicates a 4B word Mem_offset: unsigned If opcode is memset, the 32-bit {granule, address} is repeatedly written into the memory. |
|  | [30:0] | mem_offset |  |
| dst_mem_addr | [31:31] | granule | 0: address indicates a 512B word, 1: address indicates a 4B word |
|  | [30:0] | mem_offset | unsigned |
| src_stride0 | [31:0] | src_stride_0 | (512B only) signed (can be negative), in a unit of 512B, can be 0 for data duplication |
| dst_stride0 | [31:0] | dst_stride_0 | (512B only) signed (can be negative), in a unit of 512B |

Note: steps_per_stride0 is implicitly set by the length.

Descriptor size: 32B

The minimum DMA length for a transaction is always 0B. DMAs can still set the done bit in Sync Flag Memories (the count field should not be incremented).

The maximum DMA length of a transaction is limited by the size of the length field (31-bit) multiplied by the granularity (4B or 512B). In practice, it is limited by the physical size of the memory.

Striding is calculated as below:

for (int i0 = 0; i0 < length; i0++)

// read from the following source address

rd_addr = (mem_offset + i0 * src_stride_0) * granule;

# DMA Transfer Properties

## DMA Length

The DMA Length for a transaction is a non-negative integer multiple of length_granule (either 4B or 512B). DMAs with a length greater than 512B are broken into a sequence of 512B elements. Only lengths less than or equal to 64B (in multiple of 4B) or multiple of 512B are allowed. The DMA Length can be 0B. All other lengths raise a fatal error.

Each element of the transaction typically performs one or more atomic read operations from a source memory, transfers the data to a destination memory, and performs one or more atomic write operations to the destination memory. Note: if the source opcode is memset, then the data is generated by the DMA engine directly rather than accessing the indicated source memory.

The minimum DMA Length for a transaction is always 0B, i.e., it is possible to request a DMA that does not transfer any data. This is not necessarily a no-op, because DMAs can still set the “done” bit in Sync Flag Memories.

The maximum DMA Length for a transaction is limited by the size of the length field in the descriptor (31 bits, multiplied by a 4B or 512B address granularity). In practice it is limited by the sizes of the Source and Destination Memories. For example, DMAs writing to the XYS0 Smem cannot be larger than the 1MB Smem capacity.

DMAs that. Exceed the bounds of a memory address region do not wrap, and may cause references to reserved addresses. DMAs that cause reads or writes that reference a reserved address will cause fatal errors. These errors are triggered by the resulting memory access rather than by the DMA engine processing the descriptor.

## 0-Length DMAs

DMA Transactions with a length of 0 are always supported. In this case, the “count” field of the destination Sync Flag should not be incremented by the DMA engine, but it must still set the “done” bit. 0-Length DMAs can also be used to directly set a “done” bit (this can also be accomplished with an atomic write instruction) or for software convenience to allow for a general implementation whose transfer size (including a possible zero) is not known at compile time.

## Strided DMAs

Strided DMA Transactions are supported to facilitate efficient scatter-gather operations for data in 512B elements. All striding occurs at 512B intervals (even for Smem). This behavior applies even if the DMA length is less than 512B or the DMA is targeting a memory that naturally is divided at 4B boundaries (like Smem).

DMA Transactions can use one level of stride, with support of different source stride size and destination stride size.

### Stride

src/dst_stride_n is the size of the jump at the nth level of stride. Stride level 0 is the outermost stride.

Stride can be a positive or negative integer in units of 512B elements. If src/dst_stride_n is set to 0 then it will lead to repeated accesses within a transaction, which can be useful for data replication.

### Steps per stride

steps_per_stride_n is the number of times the DMA steps by stride_n before doing a stride at level n-1. It is always a non-negative number. It is legal to set it to 0 together and therefore dma_length must be 0. Otherwise it is a programming error and will result in a fatal error.

The number of steps at the outermost level of stride (steps_per_stride_0) is implicitly set by the length of the transaction and is not explicitly present as a field in the descriptor. Since DLC supports only one-level striding, here steps_per_stride_0 equals to dma_length.

## Supported DMA Sources and Destinations

This section outlines the combinations of Source Memory / Destination Memory that are allowed and disallowed for each type of DMA.

### Local and Remote DMAs

All the information required to execute a local or remote DMA is expressed using a single DMA descriptor. These descriptors can implicitly address only a subset of the overall DLC Address Space. Generally, a local/remote DMA between any pair of Source and Destination Memory is possible, with the following three exceptions.

- DHB Host Metadata FIFO (HMFs) are not reachable/addressable using local/remote DMAs either as a source or a destination.

- The instruction memories (Imem, VImem, DImem) are write-only from the DMA perspective. If specified as a source memory, they will raise fatal errors when they receive read requests from the DMA Engine.

- If there is a source/destination data-type mismatch for any DMA element, fatal errors will be raised by the Destination Memory.

### Host2Chip DMAs

Host2Chip DMAs (Infeed and Direct Write) generally abide by the same limitations as those listed above for Local and Remote DMAs except that infeeds can be configured to target the HMFs.

#### Infeed DMA Support for DHB Host Metadata FIFOs (HMFs)

Each DHB has a Host Metadata FIFO (HMF) that is a special hardware FIFO used to buffer side-band information sent by the host for the current workload running on the DHB. DMAs into HMFs are only supported in a limited set of circumstances.

- DHB HMFs are not addressable by DMA Descriptors.

- DHB HMFs are only supported as a Destination Memory by Infeed Host DMAs when the corresponding Infeed Queue is configured in the 128B HMF Mode.

  - Note: DHB HMFs are technically possible to address using Direct Write Host DMAs, but actually performing the DMA operation will likely result in incorrect DHB program execution. Although HMF’s are mapped into the DLC Address Space, they are not actually memories. If written too quickly (not properly flow-controlled), they can overflow and raise a fatal error. The correct way to push into an HMF is to configure an Infeed Queue to point its Chip Infeed Buffer (CIB) at it and use the HMF Mode; available HMF space is conveyed to the HTT’s corresponding Infeed Queue hardware using Sync Flag Updates.

### Chip2Host DMAs

Outfeed DMAs support the same set of Source Memories as local and remote DMAs. This is because the Outfeed’s source (chip-side) information is derived from a DLC Descriptor.

Debug DMAs use the data streaming over the chip’s Debug Bus as their “Source Memory” in other words, it is not possible to read or write any DLC Memory using a Debug DMA.

### Magic DMAs

Magic DMAs simply inject DLC Descriptors into the chip, and do not have sources or destinations of their own. The DMA that is executed as a result of the Matic DMA’s injection follows the same rules as any normal local, remote or chip-side portion of an Outfeed DMA.

# Chip DMA Subsystem

Chip DMA Subsystem is composed of

- FXC (Crossbar)

- HTT (DMA Manager)

- HBM Controllers

- Connectivity into all DMA targets (core, Host Interface, LYP Router)

## Overview

The diagram below represents memories as boxes. The arrows between memories represent the bandwidth of datapaths. A one-way arrow represents a full-duplex bandwidth, and a bidirectional arrow represents a half-duplex bandwidth. DMAs between memories that are not directly connected are sent on the shared datapaths in the route, which do not consume intermediate memory capacity.

In DLC chip, most levels of the memory hierarchy use a native 512B (128 x 32 bits) word size. The relative capacity and bandwidth are shown below.

|  | Capacity (in 512B) | One-Way Bandwidth (in 512B) | Bidirectional Bandwidth (in 512B) |
| --- | --- | --- | --- |
| Vreg | 256x | 20x (read) + 16x (write) | 20x (read) + 16x (write) |
| Vmem | 32Kx | 8x (shared) | 8x (shared) |
| Cmem | 64Kx | 4x | 4x (read) + 4x (write) |
| HBM | 128Mx | 2x (shared) | 2x (shared) |

### Contention Points

- Datapaths: Any access will contend the shared datapaths, e.g. a Vmem -> HBM DMA contends Vmem -> Cmem and Cmem -> HBM paths.

- Vmem load/store: Vmem DMAs can only happen when Vmem is not occupied by XYS in-pipe accesses.

- Vmem banks: Vmem has 8 banks with their own address spaces. If DMAs are not accessing these banks evenly, contention may occur within certain address spaces.

- HBM channel/bank: HBMs are split into 64 (4 x 16) pseudo-channels and 8 banks with each pseudo channel. Since each 512B word is hard-partitioned across 4 pseudo-channels, HBM accesses contend on an aggregate 128 banks.

- Control: The DMA Manager has a maximum number of DMAs it can track at any moment. DMAs beyond the limit are stalled waiting for control processing.

- Interconnect: head-of-line blocking

## Functional Descriptions

### Supported Transfer Types

- Bulk Push DMA: bulk transfer of data between local and/or remote destination.

- Scalar Push Control Message: simple atomic transfer of control messages between local and/or remote destination. Scalar messages support writes to Smem and Sync Flag updates in cores.

### Modeling Pull Transfer Model Using Push-Push

Loads from local resources cannot be initiated by remote requestors directly, however it can be modeled with two push transfers. When needed, a program running on the remote requestor can indirectly initiate a push from the local resource by:

- (1st push) sending a scalar Smem write push message to the local requestor

- Updates a destination Sync Flag in the local requestor (indicating to local requestor control thread software)

- Local requestor control thread sees the Sync Flag increment, reads information that was placed in Smem

- (2nd push) initiates a push DMA transaction back to the remote node

### Software Synchronization Primitives

The push model requires that software must ensure that all required buffers are allocated within the source and destination resources, and source addresses are available to be fetched before setting up the transfer. Hardware does not handle end-to-end credits, it only handles link-level flow control along the source-to-destination path, including back pressure at arbitrations points. This means that there will be no hardware deadlocks, and that transfers once initiated, will always reach the destination.

If software has not buffered sufficiently, the location will be overwritten.

If software sets up multiple transfers incorrectly, a software deadlock can still occur.

To enable software to track buffer allocation and transfer, hardware supports the following Sync Flag, which can be specified on every data push transfer:

- Source update: increment requestor Sync Flag number of 1KB DMA blocks read from source resource (local buffer freed for reuse).

- Destination update: increment destination Sync Flag number of 1KB DMA blocks written to destination resource (new data buffer available for processing).

- Ack update: set Sync Flag “done” bit to 1 when the DMA is complete (acknowledge write received at the destination).

These Sync primitives support the following use cases:

- Sync: hold issue as global synchronization points

- Atomic increment/decrement: to communicate end-to-end credits

- Atomic memory set: to communicate completion and flags

These Sync Flags will be presented by requestors via Sync Flag memories to provide an interface for message-based synchronization. These Sync Flags will be updated using:

- Scalar control messages passed over the network via the control VC

- Control information stripped off of bulk push DMA transfer headers, and tracked by DMA interface blocks via data VC

### Modeling Request/Response Using Acknowledgement Messages

Hardware will not support a full request/response pull model. However, hardware does support generation of acknowledgement messages from the data push destination resource back to the original requestor. Within the push transaction descriptor, the requestor specifies a synchronization variable address to update with the acknowledgement. The “ack required” indication and associate synchronization variable address are carried to the destination in the push transaction. When data has sunk in the destination resources, the destination will generate a control message back to the requestor (local or remote core), which will update the specified synchronization variable. The requestor control thread can wait on this synchronization variable update to model Request/Response semantics.

# OCN Protocol

The OCN protocol is the primary on-chip data transfer protocol for a DMA based memory subsystem that follows the chip DMA architecture. In addition to DMA based data transfers, the OCN protocol supports direct read and write data transfers. The OCN protocol also supports the control messages used for Sync Flag updates.

OCN is a split-transaction protocol where a DMA is broken into several phases, each of which operates mostly independently in the OCN network.

## Concepts

### Completion

A memory access is complete when its effects are visible to all components in the system. The following is a brief description of completion that should be sufficient for the simple memory ordering model supported by OCN. It assumes a common understanding of memory models and concepts and does not attempt to define the entire model.

- For memory accesses that read from a memory location, completion of the read means that the value returned by the read will not be affected by subsequent writes to that memory location.

- For memory accesses that writes to a memory location, completion of the write means that all subsequent reads from that memory location (from any component in the system) will return that write data or subsequent write data. All subsequent writes to that memory location will replace the complete access’ write data.

- Any side effects caused by a completed access will have occurred and be visible to the entire system.

## Components

The OCN interconnect is broken into a set of Nodes that communicate messages through a shared interconnect via a set of channels. The protocol supports a variety of different network topologies which are implemented below the protocol layer.

### Nodes

The protocol supports four different types of Node. A single design block might contain more than one type of node and/or more than one instance of a given type of node.

- Client: A Client Node is a Node that is responsible for creating DMA Descriptors and coordinating between DMA transfers using Sync Flags and control messages. An example of a Client Node is XYS scalar processor.

- Processor: A Processor Node is a Node that directly generates read and write commands to a Memory Node. An example of a Processor Node would be an execution unit that has load and store instructions.

- Memory: A Memory Node is a Node that has one or more memories that can be accessed by the OCN protocol. It also contains a DMA engine. An example of a Memory Node is FXC. A Memory Node is typically either a source or destination for a DMA transfer although it can also be a target for non-DMA based reads and writes.

  - As a source for a transfer to a Bridge Node / Memory Node / Processor Node, it receives a series of reads from them.

  - As a destination for a transfer from another Memory Node, it receives a Descriptor from a Client Node, performs a series of reads of another Memory Node, and sends a series of writes to its local memory.

  - As the destination for a transfer from a Bridge Node / Processor Node, it receives and performs a series of writes.

- Bridge: A Bridge Node bridges between the OCN DMA protocol and another DMA protocol (e.g. LYP). For a DMA transfer through a Bridge Node that is initiated by a local OCN Client Node, a Bridge Node will receive a Descriptor and perform a series of reads from a Memory Node. For a DMA transfer through a Bridge Node that is initiated outside of the local OCN network, the Bridge Node will receive a Descriptor or other control mechanism from the non-local network and will send a series of writes or reads to a Memory Node. An example of a Bridge Node is LYP Router.

### Channels

OCN has the following types of independent protocol layer channels. Any pair of nodes connected to the OCN network can implement a set of OCN channels. The pair might have multiple instances of a given type of channel as well as multiple types of channels.

All instances of a given type of channel combine to form at least one virtual channel. The protocol implementation may support more than one virtual channel per channel type through the interconnect. In other words, each type of channel operates independently of every other type of channel, but on a given type of channel, two separate source destination pairs might encounter performance issues due to the shared virtual channel.

- Descriptor: A Descriptor channel is used to transmit Descriptor packets from a Client Node to a Bridge Node or Memory Node.

- Command: A Command channel is used to send a read or write command packet from a Bridge Node / Processor Node / Memory Node to a Memory Node.

- Data: A Data channel is used to send a data packet from a Bridge Node / Processor Node / Memory Node to a Bridge Node / Processor Node / Memory Node. A data packet is always related to a prior Command packet.

- Response: A Response channel is used to indicate that a destination Memory Node has accepted a prior Command packet (e.g. space has been allocated for write data) or completed the access caused by a prior Command packet. The former is known as a acceptance response packet and the latter is a completion response packet.

- Message: A Message channel is used to send a message (e.g. control or Sync Flag) packet from a Bridge Node / Client Node / Memory Node to a Client Node / Bridge Node.

Along with the definition of the protocol, a flow control mechanism must be defined to meet the ordering and forward progress requirements of the protocol.

The following table describes the types of channels implemented by each type of Nodes.

|  |  | Client | Processor | Memory | Bridge |
| --- | --- | --- | --- | --- | --- |
| Descriptor | In-bound | N | N | Y | Y |
|  | Out-bound | Y | N | N | N |
| Command | In-bound | N | N | Y | N |
|  | Out-bound | N | Y | Y | Y |
| Data | In-bound | N | Y | Y | Y |
|  | Out-bound | N | Y | Y | Y |
| Response | In-bound | N | Y | Y | Y |
|  | Out-bound | N | N | Y | N |
| Message | In-bound | Y | N | N | Y |
|  | Out-bound | Y | N | Y | Y |

## Transactions

Each OCN transaction is broken into a sequence of packets that are transmitted between nodes in the system on the OCN interconnect. The protocol requires that all packets reach their destination node and are eventually processed by that node (subject to the forward progress limitations). Dropping packets in the interconnect is not permitted. A node that receives an illegal or unexpected packet must terminate the packet properly within the protocol (e.g. return credits, send data for a read, etc.) with specified behavior that typically includes raising an error interrupt request, read as zero for reads and write ignored for writes.

There are no limits in the protocol on the number of outstanding transactions or packets of a given type. The split transaction nature of the protocol is intended to enable multiple outstanding transactions and multiple outstanding packets within a transaction.

Descriptor packets are sent on a descriptor channel. Read command and write command packets are sent on a command channel. Read data and write data packets are sent on a data channel. Read response and write response packets are sent on a response channel. Message packets are sent on a message channel.

A DMA transaction is a superset of all other transactions. Local DMA and Remote DMA are slightly different within the protocol. The protocol also supports read transactions, write transactions, and message transactions. Host DMA looks like a sequence of read, write, and message transactions from the point of view of the OCN protocol.

A Local DMA transaction consists of the following packets:

- A descriptor packet from a Client Node to the destination Memory Node.

- N (N >=1) read command packets from the destination Memory Node to the source Memory Node.

- N read acceptance response packets from the source Memory Node to the destination Memory Node indicating that the source has accepted the read command.

- N read data packets from the source Memory Node to the destination Memory Node.

- N read completion response packets from the source Memory Node to the destination Memory Node indicating that the source has completed the read access

- M message packets from the destination Memory Node to Client Nodes for source and destination Sync Flag updates.

Note: it is expected but not required that read acceptance, data and completion would be merged into a single transfer in the interconnect.

A Remote DMA transaction consists of the following packets on the requesting (local) interconnect:

- A descriptor packet from a Client Node to the Bridge Node.

- N (N >=1) read command packets from the Bridge Node to the source Memory Node.

- N read acceptance response packets from the source Memory Node to the Bridge Node indicating that the source has accepted the read command.

- N read data packets from the source Memory Node to the Bridge Node.

- N read completion response packets from the source Memory Node to the Bridge Node indicating that the source has completed the read access

- M message packets from the Bridge Node to Client Nodes for source Sync Flag updates.

Note: it is expected but not required that read acceptance, data and completion would be merged into a single transfer in the interconnect.

A Remote DMA transaction consists of the following packets on the destination (remote) interconnect:

- N write command packets from the Bridge Node to the destination Memory Node.

- N write acceptance response packet from the destination Node to the Bridge Node indicating acceptance of the write command.

- N write data packets from the Bridge Node to the destination Memory Node.

- N write completion response packets from the destination Memory Node to the Bridge Node indicating completion of the write command.

- M message packets from the Bridge Node to Client Node for destination Sync Flag updates.

Note: It is expected but not required that a write acceptance response packet would be used to indicate that space has been pre-allocated for the associated write data packet.

A Read transaction consists of the following packets:

- N read command packets from the requesting Bridge / Processor Node to the source Memory Node.

- N read acceptance response packets from the Memory Node to the requesting Node indicating acceptance of the read command.

- N read data packets from the Memory Node to the requesting Node.

- N read completion response packets from the Memory Node to the requesting Node indicating completion of the read command.

Note: it is expected but not required that read acceptance, data and completion would be merged into a single transfer in the interconnect.

A Write transaction consists of the following packets:

- N write command packets from the requesting Bridge / Processor Node to the destination Memory Node.

- N write acceptance response packet from the destination Node to the requesting Node indicating acceptance of the write command.

- N write data packets from the requesting Node to the destination Memory Node.

- N write completion response packets from the destination Memory Node to the requesting Node indicating completion of the write command.

Note: It is expected but not required that a write acceptance response packet would be used to indicate that space has been pre-allocated for the associated write data packet.

A Message transaction consists of the following packets:

- A message packet from the source Bridge / Client Node to the destination Bridge / Client Node.

## Address Space

OCN supports two orthogonal address spaces: Data and Message. The data address space is shared by all memories targeted by OCN data channel transfers. It is a single contiguous address space, but it may be sparsely populated. Accesses to reserved addresses must behave as Read-As-Zeroes / Write-Illegal and must not cause the protocol to be violated (e.g. they must get a proper response and must not lead to a hardware hang).

The message address space is accessed by message channel transfers. This address space is typically shared by Sync Flags, message passed interrupts, and other control flow resources that do not fit in the DMA based address space. It is a single contiguous address space, but it may be sparsely populated. It is a write only address space from the OCN protocol point of view. The Nodes containing the message addresses typically have hardware and/or software read access to the message resource. Accesses to reserved addresses must behave as Write-Illegal and must not cause the protocol to be violated.

## Ordering

The OCN protocol complies with the DMA system ordering model. The particular ordering requirements on each channel are as follows.

Descriptor: Descriptor packets between a generating client node and the target Bridge / Memory Node (the node with the DMA engine for the descriptor) must stay in order. This enables a sequence of DMA transactions to stay in sequential execution order. There are no ordering requirements between packets in the descriptor channel that have non-identical start and end point nodes. For good performance, the protocol implementation should generally keep descriptors in sequential execution order and maintain fairness between requests from different nodes.

Command: There are no ordering requirements between packets in the command channel. For good performance, the protocol implementation should generally keep command packets in sequential execution order and maintain fairness between requests from different nodes.

Data: There are no ordering requirements between packets in the data channel. It is to be expected that certain Memory Nodes will aggressively reorder data accesses relative to the order of command packets to maximize memory bandwidth. For good performance, the protocol implementation should generally keep data packets in the order they were put into the interconnect and maintain fairness between requests from different nodes.

Response: There are no ordering requirements between packets in the response channel. It is to be expected that certain Memory Node will aggressively reorder responses relative to the order of command packets and/or data packets to maximize memory bandwidth. For good performance, the protocol implementation should generally keep response packets in the order they were put into the interconnect and maintain fairness between requests from different nodes.

Message: Message packets between a message source and message destination pair must stay in order. This enables control messages to a memory structure followed by a write to another memory structure or a structure with side effects to guarantee sequential ordering. Message packets between different source / destination pairs do not have ordering guarantees.

## Channel Dependencies

The protocol requires that each channel type operates independently within the interconnect. In order to prevent deadlock and provide forward progress guarantees, there are further requirements between the channel ports on a node.

A packet on a given channel might not make forward progress if another packet in the same channel is not able to make forward progress unless that channel implements more than one virtual channel and the two packets are allocated to two separate virtual channels. However, forward progress on a given channel must not depend on a subsequent packet on that channel making forward progress (e.g. if packet A is ahead of packet B in the channel then forward progress of A cannot be contingent on B’s forward progress).

A packet on a given channel is permitted to not make forward progress if it is targeting a node that is not accepting packets from that channel. In general, a node must guarantee eventual forward progress for all of its input channels.

### Descriptor Input Channel

A descriptor input channel might not make forward progress if a node’s DMA engine is busy with older descriptors and the node does not have capacity to store additional descriptors. The DMA engine’s forward progress (and therefore the forward progress of the descriptor input channel) is dependent on the forward progress of the node’s output channels as well as the node’s input data and response channels. A memory node must guarantee forward progress for writes to its memory initiated by its own DMA engine. A Bridge Node must guarantee eventual forward progress for transactions to the other side of the bridge.

### Command Input Channel

A command input channel might not make forward progress if a node is busy processing other commands (including waiting for write data). The forward progress of the command input channel is dependent on the forward progress of the input data and response channels as well as the output data and response channels. A memory node must guarantee forward progress for its command channel if the associated data and response channels make forward progress.

### Data Input Channel

A data input channel must make eventual forward progress. It must not depend on forward progress on any other channel.

Note: This means that an outbound read command must either pre-allocate space to accept the resulting inbound read data or be able to sink all read data without buffering.

Note This means that an outbound write data packet must have pre-allocated space in the destination node before being sent and that the associated write command packet must be guaranteed to be accepted by the destination node before the write data packet arrives at the destination node.

### Response Input Channel

A response input channel must make eventual forward progress. It must not depend on forward progress on any other channel.

Note: This means that every outbound command must pre-allocate space to accept the resulting inbound response.

### Message Input Channel

A message input channel must make eventual forward progress. It must not depend on forward progress on any other channel.

Note: This means that every node with an inbound message channel must always be able to sink message packets without any restrictions beyond the message processing bandwidth of the receiving node.

Dependency allowed on node’s output channel

| Input channel | Descriptor | Command | Data | Response | Message |
| --- | --- | --- | --- | --- | --- |
| Descriptor | N | Y | Y | Y | Y |
| Command | N | N | Y | Y | N |
| Data | N | N | N | N | N |
| Response | N | N | N | N | N |
| Message | N | N | N | N | N |

## Other Dependency Requirements

A write command must not be sent unless write data is available. This is to prevent the case where the write response returns, the write data is not available, and the data cannot be generated due to another action in the system (this is in turn inhibited from completing due to the outstanding write command). For example, if a Bridge Node needs to send a write command, it must have received all the write data from the other side of the bridge.